Solar & Wind Deployment Intelligence Platform

Data & Implementation Documentation

Environmental Data Collection · APIs · Dataset Schema · Prediction Calculations · Recommended Dataset

Every claim in this document was verified directly against the running backend source code as of this revision — not against the design spec alone. Where an earlier draft of this document described something that was not yet implemented, it has been corrected below and the correction is called out explicitly.

# 1. Purpose

This document is the technical reference for how the platform collects environmental data, which external APIs it uses, what the database schema looks like, and how collected data is turned into a solar/wind potential and site-suitability prediction. It answers the four questions a reviewer typically asks once the architecture diagram is approved: where does the data come from, what does the database actually store, how is a score calculated, and which single dataset should we point to if asked “where does your data come from?”

# 2. Implementation Workflow (End-to-End Data Flow)

The platform runs one repeating pipeline for every site. It is not redesigned per project — once it works for one site, it works for all of them.

| Stage | What Happens | Where It Lives |
| --- | --- | --- |
| 1. Site Registration | A Planner, Project Manager, or Administrator creates a Project, then registers a Site with coordinates, land area, and optional manual notes (existing infrastructure, land ownership). | Project & Site Management module |
| 2. Raw Data Ingestion | The backend automatically calls every external connector (NASA POWER, elevation, OpenStreetMap/Overpass, satellite imagery, environmental/demographic reference data) using the site's latitude/longitude — triggered on registration, and again on demand via “Refresh data.” | Environmental Data Collection Engine |
| 3. Caching | Identical API calls for the same site and date range are cached in Redis (6 hours for weather, 24 hours for elevation) so repeat dashboard loads don't re-hit the public API. | Cache layer (Redis) |
| 4. Raw Archival | The unmodified API response is archived as-is to MongoDB, and additionally to an S3-compatible data lake if one is configured, before any parsing — so it can be re-processed later if the parsing logic changes. | Secondary DB (MongoDB) + optional Data Lake |
| 5. Structured Storage | The relevant fields are parsed out of the raw payload and written into typed table rows (one row per site per day for weather). | Primary DB (PostgreSQL + PostGIS) |
| 6. Feature Computation | The Solar and Wind Potential Engines read the stored readings and compute derived metrics (panel efficiency, capacity factor, wind power density, etc.). | Solar / Wind Potential Prediction Engines |
| 7. Suitability Scoring | Five weighted sub-scores are computed from the stored data and combined into one overall score and category. | Site Suitability Scoring Engine |
| 8. Delivery | Dashboards, PDF/Excel reports, and alerts read the already-computed tables — they never call external APIs directly. | Dashboards, Reports, Notifications |

### Why this order matters

- Raw data is archived before parsing so a bug in the parser never destroys the original evidence — stage 5 can always be re-run from stage 4's archive.

- Prediction engines (stage 6) never call an external API themselves — they only read what's already stored, which keeps them fast and testable offline.

- The suitability score (stage 7) is a transparent weighted formula, not a black-box model, so any planner can see why a site scored the way it did.

# 3. How Environmental Data Is Actually Collected

Each data type is its own small connector, all following the same pattern: call the external API with the site's coordinates → cache the response → archive the raw payload → parse and store structured rows.

## 3.1 Weather & Climate

- Input: the site's latitude and longitude, plus a 7-day lookback window.

- Call: NASA POWER's daily point API, requesting six parameters in one request — solar irradiance (ALLSKY_SFC_SW_DWN), wind speed at 10m (WS10M), wind speed at 50m (WS50M), temperature at 2m (T2M), precipitation (PRECTOTCORR), and cloud cover (CLOUD_AMT).

- The 50m wind reading is used specifically for the Wind Potential Engine, since it is far closer to a real turbine hub height (~80m) than the 10m surface reading — the engine extrapolates from it using a wind-shear power law (Section 8.2).

- Deduplication: before inserting, the system checks whether a reading already exists for that site and date, so re-running ingestion never creates duplicate rows.

- Output: one Weather_Readings row per site per day.

## 3.2 Terrain, Infrastructure, and Land Cover

| Data Type | Source | How It's Collected |
| --- | --- | --- |
| Elevation & slope | Open-Elevation API (SRTM-derived) | Point lookup at the site's coordinates; slope is derived by sampling a second point ~111m north and computing percent grade over that distance. |
| Roads, substations, transmission lines | OpenStreetMap via the Overpass API | Query features within a 15km radius of the site; each is stored as a row with a feature type and distance in km. |
| Satellite imagery / land cover / vegetation | Copernicus Sentinel Hub (optional — needs a free API key) | Scene lookup for the site's location; stores cloud cover, NDVI (vegetation index), and a land-cover label. |
| Protected areas, water bodies, farmland, population/GDP context | OpenStreetMap (Overpass, 20km radius) + World Bank Open Data (country-level) | Distance-to-nearest-feature calculations for protected areas, water bodies, and farmland; country-level population density and GDP per capita from World Bank indicators. |
| Supplemental / live weather | OpenWeather API (optional, needs a key) + NOAA Forecast API (free, US-only) | Used to cross-check the NASA POWER historical data with current conditions and a short-term forecast — kept in a separate table, not blended into the scoring inputs. |

Connectors marked “not_configured” in the platform's own health check (Admin Dashboard → Data Sources) are ones that need an API key that hasn't been added yet — Sentinel Hub and OpenWeather. The system is built to report this honestly rather than silently fail or fabricate data.

# 4. APIs Used by the Platform

| API | Purpose | Key Needed? | Cost |
| --- | --- | --- | --- |
| NASA POWER (power.larc.nasa.gov) | Solar irradiance, wind speed (10m & 50m), temperature, rainfall, cloud cover — historical daily, global coverage | No | Free |
| Open-Elevation API | Site elevation and derived land slope | No | Free |
| OpenStreetMap Overpass API | Roads, substations, transmission lines, urban areas, water bodies, farmland, protected areas | No | Free |
| World Bank Open Data | Country-level population density and GDP per capita | No | Free |
| Copernicus Sentinel Hub | Satellite imagery, NDVI (vegetation), land-cover classification | Yes | Free tier available |
| OpenWeather API | Live current conditions (cross-check, not a scoring input) | Yes | Free tier available |
| NOAA Forecast API | US-region short-term weather forecast (cross-check, not a scoring input) | No | Free |

Practical note: only NASA POWER, Open-Elevation, and the Overpass API are required to get the platform fully functional end to end — none of them need a key. Sentinel Hub and OpenWeather add depth (imagery, live forecast) but the site-suitability pipeline works completely without them.

Correction from an earlier draft: Global Wind Atlas was previously listed here as a data source. It has been removed — there is no connector for it anywhere in the codebase, and it does not have a simple point-lookup REST API (it is served as raster tiles / a WCS interface), which would make it disproportionate effort for what would only ever be a secondary cross-validation signal. The wind-resource pipeline is fully served by NASA POWER's WS50M parameter.

# 5. What Data Each Role Provides and Receives

Correction from an earlier draft: the table below has been rewritten to describe what each role can actually do in the current build, not an aspirational future design. In particular, financial assumptions (CapEx/OpEx/discount rate) are entered by whoever holds analysis permission on a project (Planner on their own projects; GIS Analyst, Project Manager, or Administrator on any project) — they are not a form gated specifically to the Investor/Developer role, and there is no UI for a GIS Analyst to manually edit terrain data (it is fetched automatically).

| Role | What They Do (Input) | What They See (Output) |
| --- | --- | --- |
| Renewable Energy Planner | Creates projects and sites (own projects only); registers coordinates, land area, and manual site notes; can run the full analysis pipeline and financial modeling on their own sites. | Recommended sites, energy forecasts, suitability scores, and investment recommendations — scoped to their own projects. |
| GIS Analyst | Can trigger data refresh / analysis on any project (all connectors, scoring). Cannot create, edit, or delete a project or site. | GIS visualizations, terrain maps, environmental analytics, and site-comparison reports across the whole portfolio. |
| Project Manager | Full read/write and analysis on every project — create/edit projects and sites, enter financial assumptions, refresh data. | Feasibility reports, cost-benefit analysis (NPV/IRR/LCOE), deployment timelines, project progress — portfolio-wide. |
| Investor / Developer | No write access — read-only across the whole portfolio. | NPV, IRR, LCOE, capacity factor, and suitability scores for every project, for investment screening. |
| Government / Regulator | No write access — read-only across the whole portfolio. | Environmental impact data (protected-area/water-body proximity) and suitability categorization for every project, for compliance review. |
| Administrator | Manages user accounts and roles; connects/removes external integrations; configures API credentials via environment variables (not through the app UI). | Everything every other role sees, plus platform analytics, data-source health status, the full audit log, and system monitoring. |

# 6. Environmental Data Points, By Category

## 6.1 Weather & Climate (time-series, one row per site per day)

- Solar irradiance (kWh/m²/day)

- Wind speed at 10m (m/s)

- Wind speed at 50m (m/s) — primary input for turbine-height extrapolation

- Temperature (°C)

- Rainfall / precipitation (mm/day)

- Cloud cover (%)

## 6.2 Terrain

- Elevation (m)

- Land slope (%)

- Land area (hectares)

## 6.3 Infrastructure Proximity

- Distance to nearest road (km)

- Distance to nearest substation (km)

- Distance to nearest transmission line (km)

## 6.4 Environmental Constraints

- Distance to protected area (km)

- Distance to water body (km)

- Proximity to agricultural land (flag) — now factored into the environmental sub-score (Section 8.3)

- Distance to urban area (km)

- Vegetation index / NDVI

- Land-cover classification

## 6.5 Socioeconomic Context (country-level, supports investment scoring)

- Population density (per km²)

- GDP per capita (USD)

# 7. Dataset / Table Structure (Columns)

This is the actual relational structure the platform stores data in — verified directly against the SQLAlchemy model definitions. Each table corresponds to one stage of the pipeline in Section 2.

## 7.1 Site (core record)

| Column | Type | Meaning |
| --- | --- | --- |
| latitude / longitude | Float | Site location — also stored as a PostGIS geometry point for spatial queries |
| land_area_hectares | Float | Site size |
| elevation_m | Float | Elevation above sea level |
| land_slope_pct | Float | Terrain steepness |
| distance_to_substation_km | Float | Manual/fallback infrastructure distance, used only if OSM data isn't available yet |
| existing_infrastructure | Text | Free-text notes entered at registration |
| land_ownership | String | Ownership status |

## 7.2 Weather_Readings (time series)

| Column | Type | Meaning |
| --- | --- | --- |
| reading_date | Date | Day the reading applies to |
| solar_irradiance | Float | kWh/m²/day |
| wind_speed / wind_speed_50m | Float | m/s at 10m and 50m |
| temperature | Float | °C |
| rainfall | Float | mm |
| cloud_cover_pct | Float | % |

## 7.3 Infrastructure_Features

| Column | Type | Meaning |
| --- | --- | --- |
| feature_type | String | road / substation / transmission_line / urban_area / water_body |
| distance_km | Float | Distance from the site to that feature |

## 7.4 Environmental_Constraints

| Column | Type | Meaning |
| --- | --- | --- |
| protected_area_distance_km | Float | Distance to nearest protected zone |
| water_body_distance_km | Float | Distance to nearest water body |
| agricultural_land_nearby | Boolean (1/0) | Flag — now scored (Section 8.3) |
| urban_area_distance_km | Float | Distance to nearest urban area |
| population_density_km2 / gdp_per_capita_usd | Float | Country-level economic context |

## 7.5 Solar_Potential / Wind_Potential / Suitability_Score (computed outputs)

| Table | Key Columns |
| --- | --- |
| Solar_Potential | annual_irradiance_kwh_m2, peak_sun_hours, panel_efficiency_pct, shading_loss_pct, performance_ratio_pct, expected_energy_output_mwh_yr, capacity_factor_pct |
| Wind_Potential | average_wind_speed_ms, wind_power_density_w_m2, turbulence_intensity_pct, turbine_suitability_score, turbine_class, expected_aep_mwh_yr, capacity_factor_pct |
| Suitability_Score | resource_score, geographic_score, infrastructure_score, environmental_score, economic_score, overall_score, category |

## 7.6 Financial_Analysis (Investment Analytics)

Not present in the earlier draft — added here since it is a real, implemented table.

| Column | Type | Meaning |
| --- | --- | --- |
| technology, capacity_mw | String, Float | solar / wind / hybrid, and installed capacity |
| capex_usd, opex_usd_per_yr | Float | User-supplied cost assumptions |
| discount_rate_pct, project_lifetime_yrs | Float, Integer | User-supplied financial assumptions |
| electricity_price_usd_per_mwh | Float | User-supplied tariff assumption |
| npv_usd, irr_pct, lcoe_usd_per_mwh, payback_years | Float | Computed outputs (Section 8.4) |

# 8. How the Data Is Used to Predict (Calculation Approach)

The current build uses transparent, physics-based formulas rather than a trained ML model for the first prediction pass. This is intentional: it's explainable to a non-technical stakeholder, works with limited historical data, and gives a clean baseline that a trained model can later replace one function at a time without changing the rest of the pipeline.

## 8.1 Solar Potential

- Average daily irradiance and temperature are computed from stored Weather_Readings.

- Cell temperature is estimated from ambient temperature and irradiance using the standard NOCT (Nominal Operating Cell Temperature) model, with NOCT = 45°C.

- Panel efficiency is de-rated from a 20% STC (Standard Test Condition) baseline using a typical silicon temperature coefficient of −0.35% per °C above 25°C.

- Shading loss is estimated from land slope — 0% slope gives roughly a 1% inherent loss (row self-shading); loss scales up to 20% at high slope, as a proxy for horizon obstruction.

- Performance ratio combines temperature de-rating, shading loss, and a cloud/soiling penalty, clamped to a realistic 40–88% band.

- Expected annual energy output uses a PVWatts-style formula: annual irradiance × performance ratio, per installed MWp.

- Capacity factor = expected output ÷ (8,760 hours/year), capped at 35%.

## 8.2 Wind Potential

- 50m wind readings are extrapolated to an 80m hub height using the standard wind-shear power law, with a Hellmann exponent of 1/7 (open terrain).

- Air density is adjusted for site elevation using the barometric formula.

- Wind power density = ½ × air density × (hub-height wind speed)³.

- Turbulence intensity = standard deviation ÷ mean of hub-height wind speeds, across the lookback window.

- Turbine class is assigned using IEC 61400-1 wind-speed bands (Class I–IV).

- Capacity factor uses a Rayleigh-distribution approximation (a standard first-pass estimate before a manufacturer power curve is available), clamped to a 5–55% realistic band.

- Expected annual energy production (AEP) = capacity factor × 8,760 hours, per installed MW.

## 8.3 Overall Site Suitability Score

A weighted formula combines five sub-scores, each normalized 0–100:

| Sub-Score | Weight | How It's Calculated |
| --- | --- | --- |
| Renewable Resource Availability | 35% | Average solar irradiance and hub-height wind speed, each normalized against a strong-resource benchmark (6.5 kWh/m²/day, 7.5 m/s) |
| Geographic Suitability | 25% | Continuous penalty of 4 points per 1% of land slope (reaching a 60-point cap at 15% slope), plus a flat 15-point penalty for elevation above 2,000m |
| Infrastructure Accessibility | 15% | Weighted distance to substation (50%), road (30%), and transmission line (20%); 0km = 100, 20km+ = 0 for each |
| Environmental Impact | 15% | Penalizes proximity to protected areas (scaling penalty inside 10km, steepest inside 2km), water bodies (flat penalty under 500m), and agricultural land nearby (flat penalty, added in this revision) |
| Economic Feasibility | 10% | Derived from IRR and LCOE once a financial analysis has been run for the site; falls back to the infrastructure-proximity score beforehand |

The five weighted sub-scores sum to the Overall Deployment Score, which is mapped to a category: Excellent (≥85), Highly Suitable (≥70), Moderately Suitable (≥55), Low Suitability (≥35), or Unsuitable (below 35). Because each sub-score is a simple, auditable rule against stored columns — not a black box — any of the scoring functions can be swapped for a trained model later without touching the weighting logic.

## 8.4 Investment Analytics (Financial Modeling)

- Net Present Value (NPV): standard discounted cash flow — initial CapEx as a negative cash flow at year 0, followed by annual net cash flow (energy revenue minus OpEx) for each year of the project lifetime, each discounted by the user-supplied discount rate.

- Internal Rate of Return (IRR): the discount rate at which NPV equals zero, solved numerically from the cash flow series.

- Levelized Cost of Energy (LCOE): total lifetime cost (CapEx plus total OpEx over the project lifetime) divided by total lifetime energy output.

- Payback period: the year at which cumulative net cash flow first turns positive, interpolated to a fractional year.

# 9. Recommended Dataset

Your mentor's note that no single suitable dataset was found is a completely normal outcome — no public dataset covers solar, wind, terrain, infrastructure, and land cover all at once. Even large commercial renewable-siting platforms combine several sources, the same way this project's architecture already does. But if the goal is one primary dataset to anchor the project on — the one thing to point to when someone asks “where does your data come from?” — the strongest choice, and the one this platform is already built around, is:

NASA POWER (Prediction Of Worldwide Energy Resources)

- Covers both solar AND wind in one dataset: solar irradiance, wind speed at multiple heights, temperature, rainfall, and cloud cover — exactly the six variables this project's Environmental Factors list asks for.

- Global coverage, daily resolution, with records going back decades — enough history to compute meaningful annual averages for any site on Earth.

- Purpose-built for renewable energy: the “RE” (Renewable Energy) community parameter set was designed specifically for solar/wind resource assessment, not adapted from general climate data.

- Free, no API key, no meaningful rate-limit wall for this project's usage pattern — removes the single biggest blocker (“couldn't find/access a dataset”) since there's nothing to register for.

- Already the dataset this codebase is built around and verified against in this document — recommending it isn't a new integration, it's documenting what's already working.

### Recommended layering order

| Priority | Dataset | Adds |
| --- | --- | --- |
| 1 (required) | NASA POWER | Solar + wind + climate — the Resource Availability score (35% weight) depends entirely on this |
| 2 | Open-Elevation (SRTM-derived) | Elevation & slope — feeds the Geographic Suitability score (25% weight) |
| 3 | OpenStreetMap (Overpass API) | Roads, substations, transmission lines, protected areas, water bodies — feeds Infrastructure (15%) and Environmental Impact (15%) |
| 4 (optional) | Copernicus Sentinel Hub | Satellite imagery, NDVI, land cover — refines the Environmental Impact score further |

This sequencing means the project is demonstrably functional — and scoreable end-to-end — after integrating just NASA POWER and one free elevation source, which is a realistic milestone target rather than waiting to find one dataset that does everything.

# 10. Summary

- Workflow: register site → pull external data → cache + archive raw → store structured rows → compute solar/wind potential → compute weighted suitability score → surface on dashboards.

- Core data source: NASA POWER (solar + wind + climate), supplemented by a free elevation API and OpenStreetMap for terrain, infrastructure, and environmental constraints.

- Storage: PostgreSQL + PostGIS as the structured store, with raw API payloads archived separately (MongoDB + optional data lake) so nothing is lost to a parsing bug.

- Prediction: explainable physics formulas today (NOCT, PVWatts-style output, wind-shear extrapolation, Rayleigh capacity-factor approximation, IEC turbine classing), designed so any single formula can be replaced by a trained ML model later without changing the rest of the pipeline.

- Suitability score: a transparent 35/25/15/15/10 weighted formula across resource, geography, infrastructure, environment, and economics — now including an agricultural-land penalty that was previously collected but not scored.

- Two corrections from the earlier draft, applied in this revision: Global Wind Atlas was removed from the API list (not implemented, disproportionate effort for a cross-validation-only signal), and the role data table was rewritten to describe actual current behavior rather than a future aspirational design.

# 11. Change Log — What Changed Since the Last Revision

Logged here for traceability, separate from the Section 9/10 documentation corrections above, which were about fixing what this document claimed — these are changes to the running application itself.

| Change | What It Affects | Why |
| --- | --- | --- |
| All 6 roles are now self-service at registration | Authentication & Access Control | The staff-PIN requirement for GIS Analyst, Project Manager, and Administrator was removed as a deliberate simplification. This is a real security tradeoff — anyone can now self-register as Administrator — not a cosmetic change, and should be revisited before any real/public deployment. |
| Google Sign-In fully removed | Authentication | Removed end-to-end (frontend button, callback route, backend OAuth endpoints, and the database table behind it) rather than left half-configured. |
| GIS View rebuilt with 4 tabs | GIS Analyst Dashboard | Previously a single map with no terrain, environmental, or comparison views. Now covers GIS Visualization, Terrain Maps, Environmental Analytics, and Site Comparison — matching the GIS Analyst Dashboard module in the architecture spec. |
| Agricultural land now scored | Site Suitability Scoring Engine | Fixes the gap identified during fact-checking (Section 8.3): the data was already being collected but never used in the Environmental Impact sub-score. |
| Full codebase verification pass | Whole platform | Every backend file compiles; every frontend import resolves; all 58 backend API routes were cross-checked against all 39 frontend calls with zero mismatches; Docker Compose configuration and dependency pins validated. |